10

| Host Mode Option | 73 | Date | Nov. 21st, 2023 | Revision | 10 |
| --- | --- | --- | --- | --- | --- |
| Mode Control Class Public Mode – No Permission Required to Use |  |  |  |  |  |

Purpose

Host mode option 73 is to support the Thin Provisioning function and the Offload Data Transfer (ODX) function (*1) provided by Windows Server 2012 or later.

*1: The Offload Data Transfer (ODX) function is supported from RAID700 70-06-09-00/00 and higher, RAID800 80-01-22-00/00 and higher, and HM700 73-03-08-00/00 and higher.

| Model |  | Support/Unsupport | Default | Class | List Page |
| --- | --- | --- | --- | --- | --- |
| DW900 | E1090, E1090H | Support | OFF | Public | 1-3 |
|  | E990 |  |  |  |  |
|  | E790, E790H E590, E590H |  |  |  |  |
| DKC910I |  | Support | OFF | Public | 1-3 |
| DW850 | G370, G700, G900, F370, F700, F900 | Support | OFF | Public | 1-3 |
|  | G350, F350 |  |  |  |  |
|  | G130 |  |  |  |  |
| DW800 | G800, F800 | Support | OFF | Public | 1-3 |
|  | G400, G600, F400, F600 |  |  |  |  |
|  | G200 |  |  |  |  |
|  | N800 | Support | OFF | Public | 1-3 |
|  | N400, N600 |  |  |  |  |
| DKC810I |  | Support | OFF | Public | 1-3 |
| DW700 |  | Support | OFF | Public | 1-3 |
| DKC710I |  | Support | OFF | Public | 1-3 |
| DKC610I/615I |  | Not Support | - | - |  |

Background

Windows Server 2012 or later supports the Thin Provisioning function and the Offload Data Transfer (ODX) function.

In order to enable the two functions on DKC, DKC needs to support new commands and responses for Inquiry Page.

However, other hosts may receive some significant impacts if DKC modifies response values such as Inquiry Page for only the two functions on all DKC ports because Inquiry Page can be referred from all hosts.

Therefore, DKC prepares a new host mode (HMO73) which enables or disables responding values of the two functions on each host group.

<DKC710I>

Available Micro Version

70-05-02-00/00 and higher

<DW700>

# Available Micro Version

73-02-01-00/00 and higher

<DKC810I>

Available Micro Version

80-01-22-00/00 and higher

<DW800> <DW850> <DKC910I> <DW900>

# Available Micro Version

All versions

Function

When HMO73 is ON, DKC enables new functions for Thin Provisioning and Offload Data Transfer (ODX) (*1) on Windows Server 2012 or later.

10

*1: The Offload Data Transfer (ODX) function is supported from RAID700 70-06-09-00/00 and higher, RAID800 80-01-22-00/00 and higher, and HM700 73-03-08-00/00 and higher.

Mode 73 = ON:

For other than Inquiry Page B2h

- LOG SENSE command is enabled.

- UNMAP command is enabled.

- Inquiry standard ANSI version is SPC4(0x06).

- Inquiry Page 8Fh is supported.

- MAXIMUM UNMAP LBA COUNT value of Inquiry Page B0h

- RAID700:

For versions earlier than 70-05-17-00/00: 0xFFFFFFFF

For 70-05-17-00/00 and higher: 0x00000200 (*2)

- HM700:

For versions earlier than73-02-02-00/00: 0xFFFFFFFF
For 73-02-02-00/00 and higher: 0x00000200 (*2)

- RAID800, HM800 and later models: 0x00000200 (*2)

- UGAVALID bit of Inquiry Page B0h is enabled.

- MAXIMUM WRITE SAME LENGTH of Inquiry Page B0h is 0x00080000 (256 MB)

- THRESHOLD EXPONENT of Inquiry Page B2h is enabled.

- UNMAP of Inquiry Page B2h is supported (LBPU＝ON).

- SNSkey/code of check condition due to pool full status (permanent or temporary) is switched.

Permanent: SNSkey/code=07/2707

Temporary: SNSkey/code=02/0414

- Check condition for warning of threshold exceedance is responded.

- DRU attribute setting when a pool is depleted.

- Read Capacity TPE bit ON.

- Read Capacity LBPRZ bit ON.

- POPULATE TOKEN command is enabled (RAID700 70-06-09-00/00 and higher, RAID800 and later models).

- RECEIVE ROD TOKEN INFORMATION command is enabled (RAID700 70-06-09-00/00 and higher, RAID800 and later models).

- WRITE USING TOKEN command is enabled (RAID700 70-06-09-00/00 and higher, RAID800 and later models).

*2: By setting MAXIMUM UNMAP LBA COUNT value of Inquiry Page B0h to 0x0200, the maximum range that can be specified by one UNMAP command can be limited to 0x0200 (= 256 kB). This can reduce the processing time and the impact on other host I/Os by the UNMAP command.

10

For Inquiry Page B2h：

< Earlier than the versions (*3) of improvement for non-virtual volumes >

- THRESHOLD EXPONENT of Inquiry Page B2h is enabled.

- LBPU=ON (UNMAP support) of Inquiry Page B2h

< The versions and later (*3) of improvement for non-virtual volumes >

[Virtual volume]

- THRESHOLD EXPONENT of Inquiry Page B2h is enabled.

- LBPU=ON (UNMAP supported) of Inquiry Page B2h

[Non-virtual volume]

- THRESHOLD EXPONENT of Inquiry Page B2h is disabled.

- LBPU=OFF (UNMAP unsupported) of Inquiry Page B2h

*3:

RAID800: 80-06-95-00/00

RAID900: 90-09-03-00/00, 90-09-23-00/00

HM800: 83-05-50-x0/00, 83-06-22-x0/00

HM850: 88-08-12-xx/00

HM900: 93-07-03-x0/00, 93-07-22-x0/00

Mode 73 = OFF (default):

For other than Inquiry Page B2h

- LOG SENSE command is disabled. (regarded as unsupported command)

- UNMAP command is disabled. (regarded as unsupported command)

- Inquiry standard ANSI version is not switched.

- Inquiry Page 8Fh, B0h, B2h is not supported.

- SNSkey/code of check condition due to pool full status (permanent or temporary) is not switched.

OFF: SNSkey/code=07/2200

- Read Capacity TPE bit OFF

- Read Capacity LBPRZ bit OFF

- Check condition for warning of threshold exceedance is not responded.

- POPULATE TOKEN command is disabled. (regarded as unsupported command. RAID700 70-06-09-00/00 and higher, RAID800 and later models).

- RECEIVE ROD TOKEN INFORMATION command is disabled. (regarded as unsupported command. RAID700 70-06-09-00/00 and higher, RAID800 and later models).

- WRITE USING TOKEN command is disabled. (regarded as unsupported command. RAID700 70-06-09-00/00 and higher, RAID800 and later models).

For Inquiry Page B2h：

- Inquiry Page B2h is not supported.

Category

- OPEN or Mainframe (applies to either or both)

OPEN

- Program Product Type (affects none, all or some such as TC, SI, UR, etc)

Dynamic Provisioning

- OS Type (affects none, all or some such as Windows, AIX, HP-UX, etc)

Windows (0x0C), Windows Extention (0x2C)

- Hardware Type (affects none, all or some such as HDD, cache, DKA or Backend related)

None

Activation Concerns

None

Document

ECN DKC710I-M135

ECN DKC710I-M168

ECN DKC710I-M187

10

ECN DW700-M013, M015, M042

ECN DKC810I-M006

ECN DKC810I-M035

ECN DKC810I-M252

ECN-DW800-M208

ECN-DW800-M209

ECN DW850-M120

ECN DKC910I-M128

ECN DKC910I-M129

ECN DW900-M084

ECN DW900-M085

Special Direction

- This option is applied when Thin Provisioning and Offload Data Transfer (ODX) (*1) are used on Windows Server 2012 or later.

*1: The Offload Data Transfer (ODX) function is supported from RAID700 70-06-09-00/00 and higher, RAID800 80-01-22-00/00 and higher, and HM700 73-03-08-00/00 and higher.

MCU or RCU

None

Side-Effects

None

Note

6

- With R800 earlier than V02+2 or HM800 earlier than V01+1, when this option is set to ON, set SOM729 to OFF (to respond with 0x7/0x2707 (SPACE ALLOCATION FAILED WRITE PROTECT) while the pool is full). If SOM729 is set to ON, the host may not recognize pool full and may not output an error message.

- After setting HMO73 to ON, make sure to perform an operation (such as a server reboot) leading to reissuing an Inquiry command from the host.

- When the VXIO driver of Veritas InfoScale 8.0 or later version is used on the Windows OS host, also set HMO114 to ON.

- The following table lists the differences in specification between HMO73 and HMO110.

When both of the options are set to ON, the setting of HMO73 is prioritized over that of HMO110.

| Functions | HMO73 | HMO110 |
| --- | --- | --- |
| LOG SENSE command | Support | Not support |
| UNMAP command | Support | Not support |
| Inquiry Standard ANSI version SPC4 (0x06) response | Support | Support |
| Inquiry Page 8Fh | Support | Support |
| Inquiry Page 80h UGAVALID bit | Support | Support |
| 0x00080000 (=256MB) for Inquiry Page B0h MAXIMUM WRITE SAME LENGTH | Support | Support |
| 0x00000200 for Inquiry Page B0h MAXIMUM UNMAP LBA COUNT | Support | Not support |
| Inquiry Page B2h THRESHOLD EXPONENT | Support | Not support |
| Inquiry Page B2h UNMAP support (LBPU＝ON) | Support | Not support |
| SNSkey/code switch at check response due to pool depletion (temporary/permanent) | Support | Not support |
| Check response when pool depletion threshold is exceeded | Support | Not support |
| DRU attribute setting when pool is depleted | Support | Not support |
| Read Capacity TPE bit ON | Support | Not support |
| Read Capacity LBPRZ bit ON | Support | Not support |
| UNMAP command | Support | Not support |
| POPULATE TOKEN command | Support | Support |
| RECEIVE ROD TOKEN INFORMATION command | Support | Support |
| WRITE USING TOKEN command | Support | Support |

6